# Parkes / GASS — FRO Parser Technical Notes

Francesco Radio Observatory (FRO) — HALO

Bolzano, Italy — lat 46.4946°, lon 11.3353°, alt 334 m

# 1. Survey Overview

GASS (Galactic All-Sky Survey) is the 21-cm HI survey of the entire southern sky (δ < 1°) carried out with the Parkes 64-m radio telescope of ATNF (Australia Telescope National Facility, CSIRO), located at Parkes, New South Wales, Australia. The parser parkes_gass_to_fro.py converts GASS data in SDFITS format into FRO HDF5 v17 format. GASS is the reference HI survey for the southern sky, complementary to EBHIS (northern hemisphere). Reference: McClure-Griffiths et al. 2009, ApJS 181, 398.

# 2. The Parkes Telescope

Key parameters:

- Type: 64-m paraboloid, solid surface

- Site: Parkes, New South Wales, Australia — altitude ~392 m a.s.l.

- Coordinate: lat −32.9984°, lon 148.2635°

- Operatore / Operator: ATNF (Australia Telescope National Facility, CSIRO)

- Frequency range: 700 MHz – 26 GHz (with various receivers)

- Beam FWHM at 1.4 GHz (multibeam receiver): 14.0' (central beam), 14.1' (beams 2–7), 14.5' (beams 8–13); GASS effective beam: ~16' after OTF gridding

- Data format: SDFITS (same standard as GBT/Arecibo)

21-cm multibeam receiver (ATNF Multibeam):

- 13 feeds in hexagonal configuration (1 central + 6 inner + 6 outer), dual linear polarisation per feed

- Operating band: 1.23–1.53 GHz (300 MHz bandwidth); cryogenically cooled

- System temperature: Tsys ≈ 21–33 K (varies by beam and frequency) — excellent for HI surveys

- GASS backend: digital correlator, 4096 channels over 8 MHz (resolution ~0.82 km/s at 1420 MHz)

- Separation between adjacent beams: 29.1'; receiver angle at 19.1° with respect to scan direction

# 3. GASS Technical Characteristics

- Frequency: HI 21-cm line, rest frequency 1420.405752 MHz

- Coverage: southern sky δ < 1° (complementary to EBHIS δ > −5°)

- Velocity coverage: −400 to +500 km/s LSR

- Velocity resolution: ~0.82 km/s per channel (4096 channels over 8 MHz)

- Angular resolution: effective beam ~16' after OTF gridding (native beam ~14')

- Nyquist step: ~7' (~0.12°) for the native 14' beam

- Noise (rms): ~57 mK per 0.82 km/s channel (final data product)

- Integration per dump: ~5 s (OTF mode, scan at 1°/min)

- Stray radiation correction: yes (included in final GASS product)

- Reference: McClure-Griffiths et al. 2009, ApJS 181, 398

# 4. Nyquist Sampling and Grid Choice

The native Parkes multibeam receiver beam at 1.4 GHz is ~14'. The GASS effective beam after OTF gridding is ~16'. The minimum Nyquist step to correctly sample all available angular information is ~7' (~0.12°) for the native beam. Parkes has an angular resolution approximately 2 times finer than Effelsberg/GBT at the same frequency, and approximately 20 times finer than SALSA Torre. The cryogenic Tsys (~21–33 K vs ~300 K for SALSA) provides significantly superior sensitivity for HI observations.

# 5. Accessing GASS Data

ATNF Archive (reduced GASS data):

www.atnf.csiro.au/research/HI/gass/ — reduced GASS data publicly available. HI cubes can be downloaded in FITS format.

NASA FITS Registry (FRO sample file):

fits.gsfc.nasa.gov — the Parkes_GASS.fits file was downloaded as a reference example of the Parkes SDFITS format, not as a primary scientific dataset. Contains 28 spectra (7 beams × 4 integrations), GLon~271°, GLat~−45°.

# 6. Parser: parkes_gass_to_fro.py

Location: ~/FRO/Parsers/Parkes/

How it works:

- Input format: SDFITS (same standard as GBT); one .fro file per beam

- WCS frequency axis per row (CRVAL1/CRPIX1/CDELT1), linear

- Polarisation from CRVAL4 (standard FITS codes)

- Measured Az/El coordinates (AZIMUTH/ELEVATIO); GLon/GLat via Astropy SkyCoord

- Environment: TAMBIENT, PRESSURE, HUMIDITY → FRO v17 Environment group

- flag_track = −1 (unknown): Parkes SDFITS has no direct tracking flag

- Backup: ~/FRO_Backup/parkes_gass_to_fro_pre_surveys.py

Output directory structure (FRO standard model):

Parsers/Parkes/Surveys/Parkes_Survey_GASS/

Parkes_Profiles_GASS/  ← raw SDFITS files

Parkes_FRO_GASS/       ← .fro HDF5 v17 output files (1 per beam)

Parkes_Plots_GASS/     ← PNG plots

# 7. Scientific Notes

- GASS is the southern counterpart of EBHIS: together they cover the entire HI sky with similar angular resolution (~14–16').

- The Parkes cryogenic Tsys (~21–33 K) is ~10 times lower than SALSA (~300 K), providing significantly superior sensitivity for HI observations at equal integration time.

- The FRO sample file (28 spectra, 7 beams, GLon~271°, GLat~−45°) covers a region of the southern sky inaccessible from SALSA Torre (Onsala, lat 57°N) and not covered by EBHIS.

# 8. Reference

McClure-Griffiths, N.M., Pisano, D.J., Calabretta, M.R., Ford, H.A., Lockman, F.J., Staveley-Smith, L., Kalberla, P.M.W., Bailin, J., Dedes, L., Janowiecki, S., Gibson, B.K., Murphy, T., Nakanishi, H., & Newton-McGee, K. (2009). GASS: The Parkes Galactic All-Sky Survey. I. Survey Description, Goals, and Initial Data Release. ApJS, 181, 398.

www.atnf.csiro.au/research/HI/gass/

Document generated: July 2026 — Francesco Radio Observatory / HALO