FRO / HALO

SALSA — Data Structure

Francesco Di Giovanni — Bolzano, Italia — August 20, 2026 — v2.0

# 1. Introduction

SALSA (Such A Lovely Small Antenna, or "Sicken Attans Liten Söt Antenn" in Swedish) is a set of three 2.3-metre parabolic dish radio telescopes located at Onsala Space Observatory (OSO), lat 57°24'N lon 11°55'E, elevation ~5 m a.s.l., Sweden, operated by Chalmers University of Technology. The three telescopes are named Torre, Vale and Brage and are freely accessible via web browser at any time and in any weather.

The FRO/HALO project uses SALSA for Galactic HI 21-cm line observations. SALSA is the only FRO telescope directly accessible from Bolzano without an observing proposal. FITS data are imported into the unified FRO v17 (HDF5) format via the salsa_fits_to_fro.py parser.

# 2. Telescope and Receiver

The three telescopes have in-principle identical hardware; differences in cable lengths and connections. The three telescopes have in-principle identical hardware; differences in cable lengths and connections.

## 2.1 Mechanical and Optical Parameters

- Dish diameter: 2.3 m (perforated surface with ~5 mm holes to reduce wind load; practical frequency limit <6 GHz)

- Operating frequency: 1350–1620 MHz (instantaneous bandwidth up to 25 MHz, user-selectable)

- Beam FWHM: ~6° at 21 cm (1420 MHz), measured on the Sun

- Rotor: SPID BIG RAS/HR, 0.1° resolution, 1.5°/s rotation speed; MD01 controller (ROT2PROG protocol over TCP)

- Pointing limits: azimuth 0–360°; a target whose azimuth crosses due north (0°/360°) cannot be tracked straight through — the telescope must unwind nearly a full turn

- Minimum elevation (absolute): 5°; recommended: > 15° for Galactic surveys, > 20° for extragalactic observations (reduces ground pickup)

- Site coordinates: lat 57°24'N, lon 11°55'E, elevation ~5 m a.s.l.

## 2.2 Receiver Chain

- Feed: ~5 cm metal probe inside the horn, sensitive to a single linear polarisation

- LNA: custom AGO build (Tommy Henderson, USA), with integrated 1350–1620 MHz filter; powered 12V DC via Bias-T on the RG-400 coaxial cable (~10 m)

- Rack pre-amplifier: Radio Astronomy Supplies WBLNA (1–2 GHz, 15 dB gain, 0.6 dB NF)

- Sampler: USRP N210 (Ettus Research) with DBSRX2 daughterboard (0.8–2.3 GHz); 14-bit ADC; via 1 Gbps Ethernet

- Backend: 8192-point FFT on the control server; bandwidth and spectral resolution user-selectable

- System temperature: Tsys ≈ 300 K (calibrated by comparison with GASS/LAB survey convolved to the 6° beam) — approximate calibration, absolute scale to be treated with caution

## 2.3 Operating Modes

- Raw mode: total power — recommended for all FRO observations

- Frequency Switched mode: not recommended for extragalactic observations (non-linearity from frequency response → spectra not stackable)

- Standard FRO configuration for Galactic HI survey: Freq. switched, centre 1420.4 MHz, ref. freq. 1417.9 MHz (throw = 2.5 MHz = BW), 4096 channels, BW 2.5 MHz, RFI MAD-σ filter enabled

- Standard FRO configuration for extragalactic observations (M31, M33): Raw, centre 1420.405 MHz, BW 5 MHz, 8192 channels → resolution 610.4 Hz/channel (~0.13 km/s at 1420 MHz)

- Position switching for extragalactic observations: ~10° azimuth offset, same elevation (recommended by Eskil Varenius, SALSA Support)

## 2.4 Nyquist Sampling and Grid Choice

SALSA Torre's beam FWHM at 1.4 GHz is ~6°. The minimum Nyquist step to correctly sample all available angular information is ~2.5–3°. FRO survey steps are 5° (fast surveys, acceptable undersampling for large-scale Galactic structure) and 2.5° (fine survey, at the Nyquist limit — optimal choice to exploit the full angular resolution of the instrument).

# 3. SALSA FITS Format

Each SALSA observation produces one FITS file per spectrum, with a single primary HDU. Key header fields:

TELESCOP  — telescope name (e.g. "Torre", "Vale", "Brage")

ORIGIN    — institution (e.g. "Onsala Space Observatory")

DATE-OBS  — UTC timestamp in ISO 8601 format

OBSTIME   — integration time in seconds

NAXIS1    — number of spectral channels

CRVAL1    — reference frequency in Hz

CRPIX1    — reference pixel (frequency axis)

CDELT1    — frequency step in Hz/channel

CRVAL2    — Galactic longitude GLON in degrees

CRVAL3    — Galactic latitude GLAT in degrees

BUNIT     — "K" = nominal antenna temperature (fixed Tsys ≈ 300 K assumed)

Note: FITS files produced by SALSA generate an Astropy warning regarding header padding ("null bytes"). The parser suppresses this known warning as it does not affect data reading.

# 4. Parser: salsa_fits_to_fro.py

Path: ~/FRO/Parsers/SALSA/salsa_fits_to_fro.py

## 4.1 Workflow

1. Zenity dialog to select the directory containing the FITS files (fallback to ~/FRO/Parsers/SALSA/)

2. Lists all .fits files found with DATE-OBS timestamp and pointing (GLON/GLAT)

3. Interactive file selection (printer-style syntax: e.g. "1-27" or "3,5,7-12")

4. Frequency axis consistency check across all selected files (NAXIS1, CRPIX1, CRVAL1, CDELT1)

5. Interactive metadata collection (facility, provider, instrument, survey, dataset, observation_id) with defaults suggested from the FITS header

6. Az/El computed from GLON/GLAT + DATE-OBS via Astropy (labelled "computed", not telemetry)

7. Writes FRO v17 with one spectrum per FITS file in chronological order; prints read_fro_summary()

## 4.2 Technical Details

- Frequency axis reconstruction: f(bin) = CRVAL1 + (bin − CRPIX1) × CDELT1 [Hz], bin numbered from 1 (FITS convention)

- Byte order conversion: FITS data in big-endian → native conversion via .astype(np.float32)

- Pointing_source: "computed" — Az/El do not come from SALSA telemetry but are computed via Astropy AltAz at Onsala

- Metadata correction loop: current values preserved as defaults at each iteration; explicit confirmation (y/yes) required to write

- Overwrite handling: explicit confirmation required if the output file already exists

- Calibration: BUNIT="K" is nominal antenna temperature (fixed Tsys ≈ 300 K assumed) — do not use for absolute measurements

## 4.3 Auxiliary Scripts

- plot_salsa_spectrum.py — spectrum viewer with double-click on .fits file

- rename_salsa_fits.py — renames FITS files with G{lon:03d}{lat:+03d}_ prefix rounded to 5°

- build_salsa_terrain_grid.py — builds terrain grid for obstruction analysis

- view_salsa_terrain.py — 3D viewer of terrain around Torre

- view_salsa_3d.py — interactive 3D viewer

# 5. Structure of the Produced FRO v17 File

The parser produces an FRO v17 (HDF5) file with the following main datasets:

Spectra/frequencies_hz      — float64[N_ch], Hz (from CRVAL1/CRPIX1/CDELT1)

Spectra/data                — float32[N_sp, N_ch], nominal K

Pointing/az_deg             — float64[N_sp], degrees (computed)

Pointing/elev_deg           — float64[N_sp], degrees (computed)

Pointing/glon_deg           — float64[N_sp], degrees (from FITS CRVAL2)

Pointing/glat_deg           — float64[N_sp], degrees (from FITS CRVAL3)

Time/utc_iso                — str[N_sp], ISO 8601 UTC (from DATE-OBS)

Time/integration_time_s     — float64[N_sp], seconds (from OBSTIME)

Source/facility             — str, e.g. "Onsala Space Observatory"

Source/instrument           — str, e.g. "Torre"

Observatory/latitude_deg    — float64 (57.3953)

Observatory/longitude_deg   — float64 (11.9255)

Observatory/elevation_m     — float64 (5.0)

Observation/pointing_source — "computed"

Notes                       — str (free provenance text)

# 6. Known Artefacts and Masking

- DC spike at band centre: probable LO leakage. Always exclude the central channel in analysis, regardless of tuning frequency.

- RFI spike at ~+85/90 km/s: identical across different pointings — fixed-frequency RFI, NOT celestial signal. Mask in all SALSA analyses.

- RFI spike at ~+100 km/s for GLon=140°: fixed-frequency, exclude from fitting/analysis at this longitude.

- Possible additional spikes at ~+170 km/s in some pointings (e.g. l=195°, b=0°): verify recurrence before systematic masking.

- Rock obstruction (Torre telescope): anomalous signal reduction at certain azimuth ranges. See SALSA_Obs_Torre_Rock for characterisation.

- Ground pickup at low elevation: practical threshold 15° for Galactic surveys; >20° for extragalactic observations.

- Frequency-switched data NOT stackable: non-linearity from electronic frequency response. Always use position switching for extragalactic observations.

- Nominal calibration Tsys ≈ 300 K: true value varies with elevation and ambient temperature. Do not use for absolute measurements.

# 7. Scientific Notes

- Community contribution: SALSA slewing/integration bug reported by FRO/HALO → fix implemented the same day in v1.1.8 (July 2026).

# 8. Operating Procedure

1. Identify the sky region of interest (GLON/GLAT target) and verify elevation > 15° from Onsala.

2. Access the SALSA portal (salsa.oso.chalmers.se), book the Torre telescope (preferred), configure parameters and start the observation.

3. Download the produced FITS files and save them in the appropriate survey subdirectory under ~/FRO/Parsers/SALSA/Surveys/.

4. Launch the parser (SALSA_to_FRO.desktop or terminal: python3 ~/FRO/Parsers/SALSA/salsa_fits_to_fro.py).

5. Verify the summary printed to screen (read_fro_summary) and the produced .fro file.

6. Launch the visualisation script (plot_salsa_spectrum.py) to generate output plots.

# 9. Access and Contacts

- SALSA portal: salsa.oso.chalmers.se — booking and remote control. Free access, free registration.

- Support contact: Eskil Varenius (SALSA Support, Onsala Observatory) — for technical issues, position switching, advanced configurations.

- SALSA requires no observing proposal — it is the only FRO telescope directly accessible from Bolzano at any time.

# 10. References

SALSA Portal — Onsala Space Observatory, Chalmers University of Technology:

https://salsa.oso.chalmers.se

salsa.oso.chalmers.se

Full SALSA technical specifications (source for all technical data in this document):

https://salsa.oso.chalmers.se/technical

salsa.oso.chalmers.se/technical

SALSA LNA measurements and characterisation:

https://salsa.oso.chalmers.se/technical/lna

salsa.oso.chalmers.se/technical/lna

FRO/HALO project GitHub repository:

https://github.com/Digio1/FRO-HALO

github.com/Digio1/FRO-HALO

Document generated: 20 August 2026 — Francesco Radio Observatory / HALO