FRO / HALO

SALSA — Survey Notes
SALSA_Survey_Galactic_2.5deg_300s

Francesco Di Giovanni — Bolzano, Italia — August 20, 2026 — v1.0

# 1. Description and Purpose

Future high angular resolution survey of the Milky Way with 2.5° step — equal to the Nyquist step for SALSA's ~6° FWHM beam — and 300 s integration time per pointing. The directory has been created as a placeholder; the survey has not yet started. The latitude coverage is reduced compared to the 5° surveys (GLat −10° to +10°), focusing on the Galactic plane where the HI signal is strongest.

# 2. Planned Observational Parameters

- Telescope: SALSA Torre (Onsala Space Observatory)

- Angular step: 2.5° in GLon, 2.5° in GLat (Nyquist sampling for ~6° FWHM beam)

- Latitude coverage: GLat −10° to +10° (9 latitudes per longitude)

- Integration time: 300 s per pointing

- Mode: Frequency switched

- Centre frequency: 1420.4 MHz (HI line)

- Reference frequency: 1417.9 MHz (throw = 2.5 MHz = bandwidth)

- Bandwidth: 2.5 MHz

- Spectral channels: 4096 (option A) → resolution 610.4 Hz/channel (~0.13 km/s at 1420 MHz); or 8192 (option B) → resolution 305 Hz/channel (~0.064 km/s) — same resolution as the local HALO receiver, no additional cost

- RFI filter (sliding MAD-σ): enabled

- Longitude range: to be defined (probably GLon 80°–200° as per the main survey)

# 3. Status

- Status: planned — directory created as placeholder, no data acquired yet

- Planned start: to be defined based on telescope availability and results from the Galactic_5deg_120s survey

- Estimated number of pointings: ~(GLon range / 2.5°) × 9 latitudes — order of magnitude ~400–600 pointings

# 4. Notes

The 2.5° step corresponds to the Nyquist sampling for SALSA's ~6° FWHM beam, ensuring that no spatial structure above the telescope's resolution limit is lost to undersampling. The 5° surveys (Galactic_5deg_120s and FullSky_5deg_180s) are undersampled relative to the beam, but sufficient for large-scale Milky Way structure.

# 5. References

SALSA Portal — Onsala Space Observatory:

https://salsa.oso.chalmers.se

salsa.oso.chalmers.se

FRO/HALO project GitHub repository:

https://github.com/Digio1/FRO-HALO

github.com/Digio1/FRO-HALO

Document generated: August 2026 — Francesco Radio Observatory / HALO