FRO / HALO

SALSA — Survey Notes SALSA_Survey_Galactic_5deg_120s

Francesco Di Giovanni — Bolzano, Italia — August 20, 2026 — v1.1

# 1. Description and Purpose

Systematic survey of the Milky Way in Galactic coordinates with 5° step in both longitude and latitude and 121 s integration time per pointing (set value). This is the main FRO/HALO survey with SALSA, practically complete as of August 2026. It covers a significant region of the Galactic plane visible from Onsala, with latitude coverage from GLat = −20° to +20°.

Scientific purpose: construction of the longitude-velocity (l-v) diagram of the Milky Way in the HI line at 1420 MHz; comparison with reference surveys (EBHIS, LAB, HI4PI).

# 2. Observational Parameters

- Telescope: SALSA Torre (Onsala Space Observatory)

- Angular step: 5° in Galactic longitude (GLon), 5° in Galactic latitude (GLat)

- Longitude coverage: GLon 80°–200° (complete); GLon < 85° and GLon > 200° to be completed September 2026

- Latitude coverage: GLat −20° to +20°, 9 latitudes per longitude

- Integration time: 121 s per pointing (set value; actual values 119–121 s)

- Mode: Frequency switched

- Centre frequency: 1420.4 MHz (HI line)

- Reference frequency: 1417.9 MHz (throw = 2.5 MHz = bandwidth)

- Bandwidth: 2.5 MHz

- Spectral channels: 4096 → resolution 610.4 Hz/channel (~0.13 km/s at 1420 MHz)

- RFI filter (sliding MAD-σ): enabled

- Completed pointings: ~270 (25 longitudes × 9 latitudes + partial edges)

- Observation dates: 7–12 July 2026, 11 July 2026 (partial completion)

# 3. Status and Completion

- Status: practically complete (August 2026)

- To be completed: GLon < 85° and GLon > 200° — planned September 2026, when Galactic visibility allows reasonable observing hours

- FITS files present: ~270 files in SALSA_Survey_Galactic_5deg_120s/ directory

- FRO v17 file: not yet produced — conversion pending with salsa_fits_to_fro.py

# 4. ezGal Analysis — Directory SALSA_20260707_session_ezra_ezGal

The directory SALSA_20260707_session_ezra_ezGal/ contains the plots produced by ezGal (Galactic analysis module of the ezRA suite, author Ted Cline) applied to the 7 July 2026 session. The PNG files are organised by category based on the numeric prefix in the filename:

## 4.1 Survey summary plots

- ezGal510velGLon — longitude-velocity (l-v) diagram, average intensity

- ezGal516velGLonAvg — l-v diagram, weighted average

- ezGal517velGLonMax — l-v diagram, maximum intensity per pixel

- ezGal519velGLonCount — l-v diagram, spectrum count per cell

- ezGal520velGLonPolarI — l-v diagram in polar projection, intensity

- ezGal521velGLonPolarD — l-v diagram in polar projection, density

- ezGal529velGLonPolarCount — l-v diagram in polar projection, count

- ezGal530galDecGLon — Galactic declination vs longitude

- ezGal540velGLonEdgesB_1.0100, ezGal541velGLonEdges_1.0100 — l-v diagrams with highlighted edges

- ezGal559planetRot — reference planetary rotation curve

- ezGal570galArmsSun — Galactic spiral arms from Sun perspective

- ezGal580galArmsGC — Galactic spiral arms from Galactic centre perspective

## 4.2 Spectra by longitude (ezGal60x and ezGal61x)

- ezGal601–605gLonSpectra — 5 plot variants of spectra grouped by Galactic longitude

- ezGal610–614gLonSpectraCascade — 5 cascade plot variants of spectra by longitude (spectra overlaid with progressive vertical offset)

## 4.3 Individual averaged spectra (ezGal710)

37 files ezGal710gLonDegP180_NNNByFreqBinAvg — one per session pointing. Each file shows the individual spectrum averaged by frequency bin. The number NNN in the filename corresponds to the pointing index in the ezRA session. These plots allow inspection of individual spectrum quality and identification of problematic pointings (RFI, low elevation, etc.).

# 5. Operational Notes

- FITS file naming convention: "SALSA Torre, galactic (LLL.L, ±BB.B), NNNs_YYYYMMDDTHHMMSS_..."

- Minimum elevation respected: all pointings acquired with El > 15° from Onsala

- DC spike: exclude central channel in analysis (LO leakage)

- Known RFI at ~+100 km/s for GLon = 140°: fixed-frequency spike, exclude from fitting

- Nominal calibration Tsys ≈ 300 K: do not use for absolute temperature measurements

# 5. References

SALSA Portal — Onsala Space Observatory:

https://salsa.oso.chalmers.se

salsa.oso.chalmers.se

FRO/HALO project GitHub repository:

https://github.com/Digio1/FRO-HALO

github.com/Digio1/FRO-HALO

Document generated: August 2026 — Francesco Radio Observatory / HALO