Plots Description — Effelsberg/Holmberg1

FRO / HALO Project — Technical Reference

Document version: 1.1 — 24 August 2026

# 1. Visualisation Script

Script: plot_holmberg1.py — location: ~/FRO/Parsers/Effelsberg/. The script reads the .fro file (FRO HDF5 v17) produced by effelsberg_to_fro.py and generates two interactive HTML files in Surveys/Holmberg1/Plots/. The Plotly JavaScript library is loaded directly from the system path (~/.local/lib/python3.14/site-packages/plotly/package_data/plotly.min.js, version 3.7.0); no network connection is required.

Run with:

python3 ~/FRO/Parsers/Effelsberg/plot_holmberg1.py

# 2. Visualisation Technology

Both plots use Plotly.js 3.7.0 embedded inline in the HTML. Interactive features common to all outputs:

- Rubber-band zoom: drag the mouse over the plot area to select a region; the plot redraws automatically on the selected range.

- Fixed readout: a fixed box in the upper left shows in real time the coordinates (frequency or velocity + antenna temperature) of the point under the cursor.

- Zoom reset: double-click on the plot area, or click the "Reset zoom" button top-right, to return to the full view.

- Print: the readout box and Reset button are hidden in print mode (@media print).

- Responsive sizing: the plot automatically adapts to the browser window size, including on resize.

# 3. Output 1 — Full Band

Filename: EFFBG_2026-06-23_6723_02-26_P217mm-EDD_bb1_fullband_PHASE_ONOFF_1.html

Plot title: Effelsberg 100m — HOI — P217mm-EDD — Full band, PHASE/ON-OFF

X axis: Frequency (MHz), range 1200–1600 MHz — full EDD backend band.

Y axis: Antenna temperature Ta (K, Tsys≈300K, not calibrated).

Traces: Four overlaid traces: Ph1-ON pol1 (dark blue), Ph1-ON pol2 (light blue), Ph2-ON pol1 (dark red), Ph2-ON pol2 (light red). The ON/OFF distinction is derived from DATAPAR LONGOFF/LATOFF (flag_track in the .fro file). Signal/reference separation is not applied: the traces show the raw imported data.

Interpretation: The frequency structure reflects the bandpass response of the P217mm-EDD receiver and EDD backend across 400 MHz. The HI line at 1420.405752 MHz appears as a local feature near the centre of the band; comparing the phases visually identifies the on-source phase from the reference phase before any baseline subtraction.

# 4. Output 2 — HI Line Zoom

Filename: EFFBG_2026-06-23_6723_02-26_P217mm-EDD_bb1_HI_zoom_PHASE_ONOFF_1.html

Plot title: Effelsberg 100m — HOI — P217mm-EDD — HI zoom (v=141.5±600 km/s)

X axis: LSR velocity (km/s), range Vsys±600 km/s centred on Vsys=141.5 km/s (Holmberg I systemic velocity from Ott et al. 2001). The frequency-to-velocity conversion uses the true HI rest frequency: 1420.405752 MHz.

Y axis: Antenna temperature Ta (K, Tsys≈300K, not calibrated).

Traces: Same four Ph1/Ph2 × pol1/pol2 traces as the fullband plot, restricted to the velocity range shown. Dashed traces indicate pol2.

Vertical reference lines: v = 0 km/s (green dashed line): Galactic rest, unshifted HI frequency. Vsys = 141.5 km/s (purple dashed line): Holmberg I systemic velocity (Ott et al. 2001).

Interpretation: This plot allows verification that the HI emission line of Holmberg I falls within the expected velocity range. Before drawing spectral conclusions, reference-phase subtraction (Ph2 for OFF subscans) and baseline subtraction must be applied; this plot shows only the raw imported data.

# 5. File Paths

Script: ~/FRO/Parsers/Effelsberg/plot_holmberg1.py

FRO parser: ~/FRO/Parsers/Effelsberg/effelsberg_to_fro.py

FRO file: ~/FRO/Parsers/Effelsberg/Surveys/Holmberg1/Holmberg1_FRO/EFFBG_2026-06-23_6723_02-26_P217mm-EDD_bb1.fro

MBFITS source: ~/FRO/Parsers/Effelsberg/Surveys/Holmberg1/Holmberg1_MBFITS_Profiles/EFFBG_2026-06-23_6723_02-26_P217mm-EDD/

HTML output: ~/FRO/Parsers/Effelsberg/Surveys/Holmberg1/Plots/

Plotly.js library: ~/.local/lib/python3.14/site-packages/plotly/package_data/plotly.min.js (v3.7.0)

# 6. References

- FRO format specification: fro_format_v17.py (~/FRO/FRO_System/).

- Holmberg I systemic velocity: Ott J. et al. 2001 — used as reference for Vsys=141.5 km/s in the HI zoom vertical line.

- MPIfR Effelsberg 100m: https://www.mpifr-bonn.mpg.de/effelsberg

- MBFITS format specification: https://www.iram.fr/IRAMFR/GILDAS/doc/html/mbfits-html/mbfits.html